Computer Architecture Final Assessment

Team :14

Name							Sec				BN

Ahmed Mohamed Zakaria			1				4

Abdullah Ezzat					1				34

Omar Salah Aly					2				1

Mohamed Ahmed Mohamed Ahmed	2				10

Table of Contents

Not implemented modules:	4

1.Full design:	5

Instruction Set Architecture	6

Two operands:	6

one operand:	7

Memory operations:	8

Branch and Change of Control Operations:	9

Control Signals	10

Data Forwarding Unit	13

Hazard Detection Unit	17

Branch-Hazard unit:	19

Long-Fetch-Hazard unit:	20

Load-use-Hazard unit:	20

Wrong-prediction unit:	21

RET-RTI-Reset-INT unit:	21

Swap-Hazard unit:	21

Swap-use-Hazard unit:	21

PC Predictor	22

2.Analysis:	24

1. One-op testcase:	24

2. Two-op testcase:	27

3. Branch testcase:	29

4. Branch prediction testcase:	34

5. Memory testcase:	39

Project Testing:	41

# Not implemented modules:

The processor has no Memory cache system and all instructions are working

# 1.Full design:

## Instruction Set Architecture

Rsrc1	; 1st operand register

Rsrc2	; 2nd operand register

Rdst	: result register

EA	; Effective address (20 bit)

Imm	; Immediate Value (16 bit)

### Two operands:

IR0 & IR1 = ’00’.

Possible formats:

| SHL, SHR, | 5-bits opcode | 3-bits src1 | 8-bits don’t care | 16-bits Imm |
| --- | --- | --- | --- | --- |
| SWAP, | 5-bits opcode | 3-bits src1 | 3-bits src2 | 2-bits don’t care |  |
| ADD, SUB, AND, OR, | 5-bits opcode | 3-bits src1 | 3-bits src2 | 3-bits dst | 2-bits don’t care |  |
| IADD, | 5-bits opcode | 3-bits src1 | 3-bits don’t care | 3-bits dst | 2-bits don’t care |  | 16-bits Imm |

From IR0 -> IR4 “opcode”:

| Instruction | opcode |
| --- | --- |
| ADD | 00000 |
| SUB | 00001 |
| IADD | 00010 |
| AND | 00011 |
| OR | 00100 |
| SHL | 00101 |
| SHR | 00110 |
| SWAP | 00111 |

Format of src-reg code dst-reg code:

| Reg | Code |
| --- | --- |
| R0 | 000 |
| R1 | 001 |
| R2 | 010 |
| R3 | 011 |
| R4 | 100 |
| R5 | 101 |
| R6 | 110 |
| R7 | 111 |

### one operand:

IR0 & IR1 = ’01’

possible formats:

| NOP, | 5-bits opcode | 11-bits don’t care |
| --- | --- | --- |
| NOT, INC, DEC, OUT, IN, | 5-bits opcode | 3-bits src1 | 8-bits don’t care |

From IR0 -> IR4 “opcode”:

| Instruction | opcode |
| --- | --- |
| NOP | 01000 |
| NOT | 01001 |
| INC | 01010 |
| DEC | 01011 |
| OUT | 01100 |
| IN | 01101 |
| Free slot | 01110 |
| Free slot | 01111 |

Format of src-reg code dst-reg code:

| Reg | Code |
| --- | --- |
| R0 | 000 |
| R1 | 001 |
| R2 | 010 |
| R3 | 011 |
| R4 | 100 |
| R5 | 101 |
| R6 | 110 |
| R7 | 111 |

### Memory operations:

IR0 & IR1 = ’10’

possible formats:

| PUSH, POP, | 5-bits opcode | 3-bits dst | 8-bits don’t care |
| --- | --- | --- | --- |
| LDM, | 5-bits opcode | 3-bits dst | 8-bits don’t care | 16-bits Imm |
| LDD, | 5-bits opcode | 3-bits dst | 4-bits don’t care | 20-bits EA |
| STD, | 5-bits opcode | 3-bits src1 | 4-bits don’t care | 20-bits EA |

From IR0 -> IR4 “opcode”:

| Instruction | opcode |
| --- | --- |
| PUSH | 10000 |
| POP | 10001 |
| LDM | 10010 |
| LDD | 10011 |
| STD | 10100 |
| Free slot | 10101 |
| Free slot | 10110 |
| Free slot | 10111 |

Format of src-reg code dst-reg code:

| Reg | Code |
| --- | --- |
| R0 | 000 |
| R1 | 001 |
| R2 | 010 |
| R3 | 011 |
| R4 | 100 |
| R5 | 101 |
| R6 | 110 |
| R7 | 111 |

### Branch and Change of Control Operations:

IR0 & IR1 = ’11’.

possible formats:

| RET, RTI, | 5-bits opcode | 11-bits don’t care |  |
| --- | --- | --- | --- |
| CALL, JMP, JZ | 5-bits opcode | 3-bits dst | 8-bits don’t care |

From IR0 -> IR4 “opcode”:

| Instruction | opcode |
| --- | --- |
| JZ | 11000 |
| JMP | 11001 |
| CALL | 11010 |
| RET | 11011 |
| RTI | 11100 |
| Free slot | 11101 |
| Free slot | 11110 |
| Free slot | 11111 |

Format of src-reg code dst-reg code:

| Reg | Code |
| --- | --- |
| R0 | 000 |
| R1 | 001 |
| R2 | 010 |
| R3 | 011 |
| R4 | 100 |
| R5 | 101 |
| R6 | 110 |
| R7 | 111 |

## Control Signals:

Execute Signals: (3bits)

Alu Selector:

ALU_SEL = OPCODE(3 downto 0);

IO/ALU: (1 bit)

| SEL | OPCODE |
| --- | --- |
| 0 - alu | Rest |
| 1 – io | 01101 |

Out Selector: (1 bit)

| SEL | OPCODE |
| --- | --- |
| 0 – ‘Z’ | Rest |
| 1 – Rdst | 01100 |

ALU operand 2 Selector: (1 bit)

| SEL | OPCODE |
| --- | --- |
| 0 – Rsrc2 | Rest |
| 1 – IMM | 00010 00101 00110 10010 |

MEMORY SIGNALS: (7bits)

Read/Write Select: (1 bit)

| SEL | OPCODE |
| --- | --- |
| 0 – Read | 10001 10011 11011 11100 |
| 1 – Write | 10000 10100 11010 |

Value Selector: (2 bit)

| SEL | OPCODE |
| --- | --- |
| 00 – “Z” | rest |
| 01 – [Rsrc1] | 10000 10100 |
| 10 – PC | 11010 |
| 11 – FLAGS&PC | INTR |

Address Selector: (2 bits)

| SEL | OPCODE |
| --- | --- |
| 00 – 1,0 | RST |
| 01 – 3,2 | INTR |
| 10 – EA | 10011 10100 |
| 11 – SP/SP+2 | 10000 10001 11010 11011 11100 |

(SP ALU) + (SP/SP+2) Selector: (1 bit)

| SEL | OPCODE |
| --- | --- |
| 0 – ‘+’ and ‘SP+2’ | 10001 11011 11100 |
| 1 – ‘-‘ and ‘SP’ | 10000 11010 |

SP load: (1 bit)

| LOAD | OPCODE |
| --- | --- |
| 0 | Rest of them |
| 1 | 10001 11011 11100 10000 11010 |

WB SIGNALS: (4 bits)

Write Value Select: (2 bits)

| SEL | OPCODE |
| --- | --- |
| 00 – ‘Z’ | rest |
| 01 – MEM | 10001 10011 |
| 10 – EXE | 01001 01010 01011 01101 00111 00000 00001 00010 00011 00100 00101 00110 10010 |
| 11 – [Rsrc1] | Second swap |

Write Address Select: (2-bit)

| SEL | OPCODE |
| --- | --- |
| 00 – Rsrc1 | 01001 01010 01011 01101 00111 00101 00110 10001 1001د0 10011 |
| 01 – Rdst | 00000 00001 00010 00011 00100 |
| 10 – Rsrc2 | Second swap |

## Data Forwarding Unit

Forwarding unit

Rs(1st src from ID/EX)

Rt(2nd  src from ID/EX)

Forward A

Rd3(dst from EX/MEM)

Rd2(dst from ID/EX)

Forward B

Rd4(dst from MEM/WB)

EX. RegWrite

MEM.RegWrite

WB.RegWrite

| ALU operand A | ALU operand B |
| --- | --- |
| Else if ( (Rs == Rd3) and (MEM.RegWrite)) ForwardA = 01 Else if ((Rs == Rd4) and (WB.RegWrite)) Forward A =10 Else ForwardA=00 | Else if ( (Rt == Rd3) and (MEM.RegWrite)) Forward B= 01 Else if ((Rt == Rd4) and (WB.RegWrite)) Forward B=10 Else Forward B =00 |

R_Dst(to PC_Predictor)

Dst from Reg_file

Dst from IF

Special unit "inside DF unit ":

Dst from EX/MEM

Dst from MEM/WB

If(Dst(from IF)== Dst(from EX/MEM))

{

R_Dst(to PC_Predictor)= Dst(from EX/MEM)

}

Else If(Dst(from IF== Dst(from MEM/WB))

{

R_Dst(to PC_Predictor)= Dst(from MEM/WB)

}

Else R_Dst(to PC_Predictor) = Dst(from reg_file)

INTR/RESET HANDLER

If INTR_MEM == 1 and INTR_WB == 0:

Rd/Wr = 1            -- Wr

val_sel = 00

add_sel = 11

SP_load = 1

SP_Alu = 1

else If INT_WB == 1:

Rd/Wr = 0            -- Rd

val_sel = xx

add_sel = 01

SP_load = 0

SP_Alu = x

else If Reset == 1:

Rd/Wr = 0            -- Rd

val_sel = xx

add_sel = 00

SP_load = 0

SP_Alu = x

else: Rd/Wr = Rd/Wr_in     -- Rd

val_sel = val_sel_in

add_sel = add_sel_in

SP_load = SP_load_in

SP_Alu = SP_Alu_in

SWAP HANDLER

If Opcode == 00111 and Val_sel == 0:

val_sel = 100

add_sel = 1

else if opcode = 00111 and val_sel != 0:

val_sel = 101

add_sel = 0

else:

val_sel = val_sel_in

add_sel = add_sel_in

## Hazard Detection Unit

Stall_bit_1

1-bit Reg

Opcode_F

Rdst_D_code

Rdst_F_code

Branch-Hazard

Prediction_bit

Rsrc1_D_code

Opcode_D

Rsrc1_E_code

Opcode_E

Rsrc1_M_code

Opcode_M

Rdst_M_code

Rdst_E_code

Long-Fetch-Hazard

clr

Stall_bit_2

1-bit Reg

Opcode_F

Fetch-Hazard

Previous_Stall

1-bit Reg

Swap-Hazard

Opcode_D

Load-use-Hazard

Stall_bit_3

Opcode_D

Rsrc1_D_code

Opcode_E

Rsrc2_D_code

Stall_bit_6

Rdst_E_code

Swap-use-Hazard

Opcode_D

Opcode_E

Opcode_M

Stall_bit_7

Wrong-prediction

Rsrc2_D_code

Rsrc1_D_code

Rdst_E_code

Stall_bit_4

Opcode_E

Rdst_M_code

Wrong-prediction-bit

Prediction-bit

ZF

Stall_bit_5

1-bit Reg

RET-RTI-Reset-INT

Opcode_M

Opcode_F

INT_F

Falling-edge checker

INT_W

Reset_M

Reset_F

Load_ret_PC

Hazard-Detection

Unit

Stall_bit_3

Stall_bit_1

PC_Write

Stall_bit_6

Stall_bit_5

Stall_bit_7

Stall_bit_7

Stall_bit_6

Stall_bit_5

Stall_bit_4

Stall_bit_3

Control_Unit_Mux

Stall_bit_2

Stall_bit_1

## Branch-Hazard unit:

As we predict in fetch stage so if we have a jz instruction with Prediction_bit = 1 “predict taken”, a jmp instruction or a call instruction so we need to pass Rdst to the PC.

This will cause hazard in some cases if Rdst is not ready in the register file.

Case 1:

- Add R3,R1,R2

- Jz R3

- Instr. “if taken”

When “jz” is in fetch stage, “add” will be in decode stage so Rdst will not be calculated yet.

We need to stall once then forward Rdst from execute stage to the PC.

| Add R3,R1,R2 | F | D | E | M | W |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Jz R3 |  | F | stall | stall | stall | stall |  |  |
| Jz R3 |  |  | F | D | E | M | W |  |
| Instr. |  |  |  | F | D | E | M | W |

Case 2:

- LDD R3,Imm

- Instr.

- Jz R3

- Instr. “if taken”

| LDD R3,Imm | F | D | E | M | W |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instr. |  | F | D | E | M | W |  |  |  |
| Jz R3 |  |  | F | stall | stall | stall | stall |  |  |
| Jz R3 |  |  |  | F | D | E | M | W |  |
| Instr. |  |  |  |  | F | D | E | M | W |

Case 3:

- LDD R3,Imm

- Jz R3

Same as case 2

Instr. “if taken”

| LDD R3,Imm | F | D | E | M | W |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Jz R3 |  | F | stall | stall | stall | stall |  |  |  |
| Jz R3 |  |  | F | stall | stall | stall | stall |  |  |
| Jz R3 |  |  |  | F | D | E | M | W |  |
| Instr. |  |  |  |  | F | D | E | M | W |

Case 4:

- Swap R1,R2

- Jmp R1

| Swap R1,R2 | F | stall | stall | stall | stall |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Swap R1,R2 |  | F | D | E | M | W |  |  |  |  |  |
| Jmp R1 |  |  | F | stall | stall | stall | stall |  |  |  |  |
| Jmp R1 |  |  |  | F | stall | stall | stall | stall |  |  |  |
| Jmp R1 |  |  |  |  | F | stall | stall | stall | stall |  |  |
| Jmp R1 |  |  |  |  |  | F | D | E | M | W |  |
| Instr. |  |  |  |  |  |  | F | D | E | M | W |

In general: if I have jz with  taken pred. , jmp, call, I will stall once in these cases

- The previous instr. is one-op, two-op, LDM, LDD or POP that has  the Rdst as me.

- The instr. before previous one is LDM, LDD or POP that has the Rdst as me.

- One of the previous 3 instructions is swap with the Rsrc or Rdst as my Rdst

Stall_bit_1: stall the whole pipe

## Long-Fetch-Hazard unit:

There’re some instructions that are 32-bit in size so it can’t be fetched once from the memory.

So what we need is to fetch the first half and to stall this half in the decode till we fetch the second part and start decoding and to make sure that the next half will not cause this stalling even if it seems to be a 32-bit instr. cause in fact it’s not, it’s just the rest of the last instruction.

Stall_bit_2: stall decode of the next cycle only.

## Load-use-Hazard unit:

Example:

- LDD R3,Imm

- Add R1,R2,R3

| LDD R3,Imm | F | D | E | M | W |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Add R1,R2,R3 |  | F | D | stall | stall | stall |  |  |  |
| Add R1,R2,R3 |  |  | F | D | E | M | W |  |  |

Note: no register because I want to stop fetching next instruction once add get into decode stage to fetch the same instr. again.

Stall_bit_3: stall the whole pipe

## Wrong-prediction unit:

It outputs 1-bit “Wrong-prediction-bit”  (prediction-bit XNOR ZF)

If there’s a “jz” instruction in execute stage, there are two cases:

- If prediction-bit = zero-flag : Wrong-prediction-bit = 0

- If prediction-bit != zero-flag : Wrong-prediction-bit = 1

Stall_bit_4: stall decode only.

## RET-RTI-Reset-INT unit:

If you found RET, RTI instruction or interrupt or reset in fetch stall upcoming instructions till this instruction or this interrupt/reset finishes memory stage so that the new PC is ready.

Stall_bit_5: stall the whole pipe

## Swap-Hazard unit:

If there’s a swap in decode it stalls it and re-fetches it again.

To make sure that the re-fetched swap will not cause another stall, we keep track of the last stall due to swap, if it was ‘1’ so do not stall.

Stall_bit_6: stall the whole pipe

## Swap-use-Hazard unit:

Example:

- Swap R1,R2

- Add R1,R2,R3

| Swap R1,R2 | F | stall | Stall | stall | stall |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Swap R1,R2 |  | F | D | E | M | W |  |  |  |
| Add R1,R2,R3 |  |  | F | D | stall | stall | stall |  |  |
| Add R1,R2,R3 |  |  |  | F | D | stall | stall | stall |  |
| Add R1,R2,R3 |  |  |  |  | F | D | E | M | W |

Note: no register because I want to stop fetching next instruction once add get into decode stage to fetch the same instr. again.

Stall_bit_7: stall the whole pipe

## PC Predictor

If (Wrong_Prediction_bit == 0 and Load_ret_PC== 0):

if ( (opcode_F == jz) and (Prediction_bit == 1) ) or ( (opcode_F == jmp) or ( (opcode_F == call):

PC_predicted = Rdst_val

PC_UnPredicted = PC+1

If  (opcode_F == jz) and (Prediction_bit == 0):

PC_predicted = PC+1

PC_UnPredicted = Rdst_val

Else If (Wrong_Prediction_bit == 1):

PC_predicted = Unpredictted_PC_E

PC_UnPredicted = PC+1

Else If (Load_ret_PC== 1):

PC_predicted = PC_Mem

PC_Write

PC_Predicted

PC

Load_ret_PC

Wrong-prediction-bit

Unpredicted_PC_E

PC_UnPredicted = PC+1

# 2.Analysis:

## 1. One-op testcase

Without control hazard unit and forwarding unit:

Number of clock cycle = 26

Hazards:

Without control hazard unit and forwarding after add NOP to solve hazards

Without control hazard unit and forwarding before add NOP to solve hazards

Note: there is 4 data hazards

- Red: When “inc R1” enter execution stage, R1 value has not yet been updated, so it increases “00000000” instead of “FFFFFFFF”

- Blue: When “NOT R2” enter execution stage, R2 value has not yet been updated, so it inverses “00000000” instead of “00000010”

- White: When “Dec R2” enter execution stage, R2 value has not yet been updated, so it decreases “00000010” instead of “FFFFFFEF”

- Yellow: When “out R2” enter execution stage, R2 value has not yet been updated, so it out “FFFFFFFF” instead of “FFFFFFEE”

Without forwarding unit:

Same as without control hazard unit and forwarding unit

Without control hazard unit:

Number of clock cycle = 17

Hazards: non

With control hazard unit and forwarding unit:

With control hazard unit and forwarding

Number of clock cycle = 17

Hazards: non

## 2. Two-op testcase

Two operand test case

Without control hazard unit and forwarding unit:

Number of clock cycle = 27

Hazards:

Without control hazard unit and forwarding after add NOP to solve hazards

Without control hazard unit and forwarding before add NOP to solve hazards

Note: there is 2 data hazards

- Red: When “SHR  R2,3” enter execution stage, R2 value has not yet been updated, so it shifts “00000019” instead of “00000064”

- Yellow: When “SWAP R2,R5” enter execution stage, R2 value has not yet been updated, so it swaps “0000FFFF” with “00000064” instead of “0000FFFF” with “0000000C”

Without forwarding unit:

Same as without control hazard unit and forwarding unit

Without control hazard unit:

Number of clock cycle = 25

Hazards: non

With control hazard unit and forwarding unit:

With control hazard unit and forwarding

Number of clock cycle = 25

Hazards: non

## 3. Branch testcase

- (Fixed Assembly file by adding nop’s of each incremental step is attached)

- Hazards in the testcase:

  - JMP R1			- Control Hazard

  - AND R1, R5, R5		- Control Hazard Because of the Interrupt

  - JZ R2			- Control Hazard

  - JZ R3			- Control Hazard

  - NOT R5

INC R5			- Data Hazard

  - In R6

JZ R6			- Data and Control Hazard

  - RTI			- Control Hazard

  - Call R6			- Control Hazard because of the interrupt

  - RET			- Control Hazard

- Without Forwarding and Hazard Detection Unit: (71 cycles using nops)

  - All Hazards were present:

Figure 1: undefined behaviour on interrupt at 30 and instruction at 32 was fetched and excuted

Figure 2: JZ at 31 was non-taken and the JZ at 50 was taken. and wrongly fetched instructions wasn't flushed

Figure 3: Data Hazard at R5 where an old value was used

  - To solve them using no operation:

    - The JMP at 1, JZ at 3 and 4: add two nop after them to allow the new address calculation to take place without fetching wrong instructions

    - Interrupt at 2 and 8: add 4 nop after them because the interrupts happen on 2 cycles and the second cycle is like a load-use case.

    - The RTI at 7 and ret at 9: add 3 nop after them because it’s a load-use case where the PC is loaded from the memory

    - The data Hazards at 5 and 6: add two nop between the two instructions

    - The control hazard at 6: add two nop after the jump to allow the new address calculation to take place without fetching wrong instructions

- With Forwarding Unit but no Hazard Detection: (69 cycles using nops)

  - All the hazards are still present except: Data Hazard at the INC R5 at 5 is solved using forwarding:

Figure 4: Data Hazard Solved using Forwarding unit

  - Data Hazard at 6 is not solved because branch unit is the one using the R6 value not the alu.

- With Hazard Detection Unit but without the PC prediction: (67 cycles using nops)

  - The Data Hazard at 6 is handled by the stalling of the hazard unit.

  - The rest of the Hazards are still present.

- With everything on: (59 cycles using nops)

  - All the Control Hazards are solved using branch prediction and flushing:

Figure 5: the control hazard of JMP at 17

Figure 6: the control hazard of the interrupt handled by hardware stalling

Figure 7: the control hazard at 102

Figure 8: the instruction 32 flushed by "control_unit_mux" signal

Figure 9: the control hazard at 54 JZ R6 by hardware stall

Figure 10: the interrupt signal at 201 handled using hardware stall

Figure 11: the control hazard at 302 handled using hardware stalls making sure the INC R7 is not executed.

## 4. Branch prediction testcase

Times of end cycles

- with forwarding only: 16900

- without forwarding & hazard detection & flushing: 23300

- with all units working: 13700

- with forwarding and hazard detection: 16900

Waveforms:

- with forwarding only

- without forwarding & hazard detection & flushing

- with all units working

- with forwarding and hazard detection

Comments:

- No data hazard with forwarding because ldm loads imm values not from memory so the forwarding unit will forward it.

- Both with forwarding only and with forwarding and hazard detection got the same number of cycles needed to finish because in this file forwarding unit made that there’s no data hazards at all.

Hazards happened:

- Disabling flushing and always predict not taken caused control hazard after each jmp/jz/call instructions and to solve it we added 2 NOPs after each of them

Instructions got the hazard:

JMP R3

JZ R1

JMP R3

JMP R3

JZ R3

Check the testcase analysis code

- Disabling hazard detection unit caused no hazards

- Disabling both hazard detection unit and forwarding caused data hazards and to solve it we added NOPs before each instruction causing hazard to make sure that while it’s in decode stage the data are ready in write back stage

| Instructions got the hazards | Number of NOPs needed before it |
| --- | --- |
| JMP R3 | 1 |
| OUT R4 | 2 |
| JMP R3 | 1 |
| OUT R4 | 2 |
| AND R0,R2,R5 | 2 |
| OUT R4 | 2 |

## 5. Memory testcase

Without control hazard unit and forwarding unit:

Number of clock cycle = 28

Hazards:

Without control hazard unit and forwarding after add NOP to solve hazards

Without control hazard unit and forwarding before add NOP to solve hazards

Note: there is 2 data hazards

- Red: When “PUSH R1” enter execution stage, R1 value has not yet been updated, so it pushes “00000000” instead of “000000F5”

- White: When “STD R2,200” enter execution stage, R2 value has not yet been updated, so it stores “0CDAFE19” instead of “000000F5”

Without forwarding unit:

Same as without control hazard unit and forwarding unit

Without control hazard unit:

Number of clock cycle = 25

Hazards: non

With control hazard unit and forwarding unit:

With control hazard unit and forwarding

Number of clock cycle = 25

Hazards: non

# Project Testing

All do files included in folder “do-files”

Test case 1: one operand: (do file name: oneOperand_dofile.txt, included in Project Testing folder)

Test case 2: two operands: (do file name: twoOperands_dofile.txt, included in Project Testing folder)

Test case 3: branch:

Test case 4: branch prediction:

Test case 5: memory: